USE AS REF

Cage 1 instruments:

Star-ODDI: S12665, 3 min sample interval

CO2PRO: 43-214-75

CO2PRO Battery: 43-227-35

SBE Pump:

PME MiniDOT: 801016, turned on at 1619, 1 min sample interval

PME MiniDOT Wiper: 754797

CO2Pro Settings, timed, start at 2300 on 4/24, sample interval 240 min (should give ~200 days since I’ve already used ~90 days), 3 readings per sample, re-zero interval is 24 hours. Confirmed clock is good and in UTC. Started instrument.

Deployed at 1906 on April 24 at 50 deg 36.785’ N 126 deg 32.981’ W

NEED TO SETUP PME MiniDOT before deployment! Completed.

USE IN KELP

Cage 2 instruments:

Star-ODDI: S12074, 3 min sample interval

CO2PRO: 43-213-75

CO2PRO Battery: 43-228-35

SBE Pump:

PME MiniDOT: 315342, turned on at 1628, 1 min sample interval

PME MiniDOT Wiper: 289914

CO2Pro Settings, timed, start at 2300 on 4/24, sample interval 240 min (should give ~200 days since I’ve already used ~90 days), 3 readings per sample, re-zero interval is 24 hours. Confirmed clock is good and in UTC. Started instrument.

Deployed at 1741 on April 24 at 50 deg 37.284’ N 126 deg 30.976’ W

NEED TO SETUP PME MiniDOT before deployment! Completed.